PCB has 2 errors, one error - varactors D1 and D2 drawn opposite. C27 need to go to ground, place it to another leg of R22.

Here is photo attached.

More notes on assembly:

I have provided you with coils:

4+1

1 red is for first oscillator. You need to wind 44 turns.

2 out of 4 others used in RF and you need to remove built in capacitors from them. One used for sense tuning, another one in input circuit.

Sense coil need to be rewind completely with 62-65 turns.

Another one in RF - remove cap and wind secondary.

Two others are for IF - only need to wind secondary and leave caps in place.

Start assembly from AF and move to RF, check signal as you go.

Schematic is good, you do NOT need: IC4, C37, C41, C42, C50, C51

R41 - use 3k, you can adjust it, it affects main volume level

C5 - dont need it also

Parts to be installed at factory:

IC1,IC2					SA602

IC3					UA741C

IC5					PIC16F886

VR1					LP2981 	3.3v

VR2					MC78L06	6v

T1,T2					BC850C

D1,D2					NTE618

D3,D4					BAT74 uses only one part because case has 2 diodes inside

C2					6-30p

C5					100p*

C47,C34,C26,C29,C39,C48,C43,C15		10uF	electrolitic

C36					1uF	electrolitic

C35,C44,C12				1uF

C4,C33					1500p

C3,C14,C27				10n

C16,C17,C18				470p

C55					5-60p*

C22,C38,C40,C20,C25			100n

C45					940p

C46					830p

C49					120p

C23,C30					1n

C11					12p

P1					2k*

R2,R13,R39				1k*

R7					15k*

R5					220k

R6,R27,R21,R37,R38			100k

R10,R12					3k

R9					1.1k

R22					360k

R23					1M

R26					750k

R20,R31	R36				2k

R24					75k

R25					82k

R29					2.4k

R40,R11,C10,R32,C32,C1			0

R3,R17,R18,R41				10k

R35,R16					20k Pot

Parts needs to be installed manually:

R4,R28					100k Pot

L1					120uH* (60 turns on TOKO S7AG-A033HM)

L3					53 turns for secondary on TOKO S7AG-A033HM, remove capacitor from contour

L5					44 turns for secondary on TOKO S7AG-A033HM

L10					59 turns for secondary on TOKO S7AG-A033HM

L7					44 turns on T-25-2

ZQ1					455kHz

CFWLA					CFK455J or similar

C7					10n

C21					100n

*******************************************************************************

*************		This is open source project 		***************

*************	   80 Meters FoxFinder receiver by KB1RLI	***************

*************	email all questions to vadim.afonkin@gmail.com	***************

*******************************************************************************

Folders content:

1. Schematics - most recent version of receiver

2. Enclosures - drawings to manufacture receiver enclosure. Use PDF files - they are most accurate. Autocad files does not have measurments. All sizes are in inches, not in millimeters

3. FRMWR - most recent firmware for receiver

4. Parts list - parts list/BOM

5. PCB - Gerber files for PCB manufacturing

6. Photo - photos of assembled receiver, pcb, etc.

Other notes:

main.hex - firmware for MCU - PIC16F886

Current firware supports following functions:

1. AutoTone - tone will turn on based on signal level, on standard 3W TX from about 900-1k. Tone attached to signal and controlled by volume.

2. Tone pitch - attached to signal level

3. GPS/Display/Distance calculation

No changes needed if no GPS/Display used.

73!

Vadim

KB1RLI